Coastline Flight Training

Cessna 172 R

Detailed Checklist

12 Pathfinder Drive

Caloundra Airport  Qld  4551

Ph: 1800 072 118

(+61 7) 5491 8588

Web: www.chopperline.com

www.coastlineaviation.com.au

Email: info@chopperline.com

Checklist Use

The Pilot Operating Handbook is the primary reference and if any discrepancies exist between the Pilot Operating Handbook and this checklist, the Pilot Operating Handbook information takes precedence.  The Chief Flight Instructor is to be notified immediately if any discrepancy is found so that the appropriate amendment(s) to the checklist can be made.  There are two methods for checklist use...

- DO & CHECK

- READ & DO

Some normal & emergency checklist actions are required to be completed in a timely fashion.

These checklists are memory checks.

Memory checks are marked by a box and have bold type in their entirety.

This detailed checklist is primarily to be used as a READ & DO checklist.

***** 	 Indicates particular checks are complete

At all times:

AVIATE						     Fly the Aircraft

NAVIGATE					           Position the Aircraft

COMMUNICATE					       Radio & Brief

ADMINISTRATE					   Checklist Action

Hand Over Procedures:

“HANDING OVER			.........           Taking Over”

“I HAVE CONTROL			........    You have control”

CAUTION: DUE TO VIS RESTRICTIONS FM THR RWY 12 & 23,     PILOTS ARE TO ENSURE NO CONFLICT EXISTS PRIOR TO TKOF.

YCDR CTAF 118.8		BN FIS        129.0

YCDR PAL	119.6		BN RADAR 125.7

BN FLIGHTWATCH 128.75

YBMC ground	121.1

YBMC tower		124.4

YBMC ATIS		119.8 / 114.2

YBMC AWIB		119.8

MC VOR		114.2

MC NDB		380 (M _ _  C _ . _ . )

YBMC PAL/AFRU	124.4

CENSAR	         1800 814 931

Maroochy Tower –   07 5448 7662

Maroochy AWIB –   07 5450 6032 or 119.8

Brisbane ATS unit – 07 3866 3588 (slot time)

Brisbane ATS unit – 07 3866 3694 (IAL)

Pilot Recency Checks/Personal Minimums  -  Complete

PRE START

Pre flight inspection ( refer POH + AWB/AAC)		    Complete

Maintenance Release		         Checked, complete & on board

POH/GNSS Manual					    On board

PAX brief						    Complete

ATIS/ AWIB						    Received

Clock (Aircraft &/or Pilot)				     Checked

Assigned Altitude Indicator					 Set

Altimeter(s) (VFR +/- 100ft) (IFR +/- 60ft)  QNH Set & Cross Check

Transponder				             Standby & Code Set

Take Off Safety Brief (Pilot – Pilot)			          Given

Departure Brief	(VFR “Visual”) (IFR/SID SRD Plate)              Given

Approach Brief	(VFR: Not  Applicable) (IFR: Plates) 	          Given

Seats						 Adjusted & Locked

Seat belts/Harnesses				        On & Adjusted

Brakes						     Tested & Set ON

Fuel Selector Valve					            Both

Fuel Shut Off Valve				    ON, Push Full IN

Alternate Static Source					     OFF / IN

Mixture				   ICO-   Idle Cut Off, Pull Full OUT

Throttle						   Open ¼ inch/5mm

Panel Lights/ Bright Dim Switch                                                     Set

Avionics Master Switch					            OFF

Nav Lights					      As required, ON

Electrical Equipment (switches, Landing/Taxi lights)	            OFF

Flashing Beacon						ON

Circuit Breakers				             Checked IN

Avionics Circuit Breakers			             Checked IN

Master Switch 							ON

T’s & P’s ( COLD or HOT Start)			     Checked

*****

Cold Start

Aux Fuel Pump Switch					             ON

Mixture					      Full Rich (Full forward)

Suitable fuel flow (3-5 seconds)

ICO-Idle Cut OFF (Full Aft)

Aux Fuel Pump Switch					           OFF

Propeller Area Clear	    “Clear PROP”/3xflash LDG light @ night

Ignition Switch	(Key)					           Start

Mixture				           Advance (when engine starts)

*****

Hot Start

Propeller Area Clear	    “Clear PROP”/3xflash LDG light @ night

Ignition Switch	(Key)					           Start

Mixture				           Advance (when engine starts)

*****

After Start

Oil Pressure					          Check/Rising

Alternator/Ammeter					     Checked

Instrument Air (Suction/Vacuum)			     Checked

Avionics Master Switch					             ON

Panel Lights/ Bright Dim Switch 	            Not required OR Set

Intercom					        Set & Checked

Marker Beacons				     Tested & Set LO

Radios					ON/Volume Checked & Set

Navaids					      Set & Identified

Transponder				            Code Set & Standby

Flaps							       Retract

Taxi Light 						             ON

AI/AH					           Erect & Pitch Bar Set

DG/DI (HSI)					    Checked/Aligned

ATIS/AWIB						    Received

Taxi Call       		  				   Complete

Park Brake						            OFF

*****

Taxi

Brakes							     Checked

Rudder							     Checked

Flight Instruments & Navaids	(Function)		     Checked

*****

Run Up

Park Brake						            Set/ON

Clear Behind						              Check

Engine Instruments (T’s & P’s)			          Checked

Throttle					             Set 1800RPM

Magnetos					   	              Check

“Right Actual ___”   “Left Actual ___”   “Differential ___”

Vacuum/Suction					          Checked

Ammeter						          Checked

Annunciator Panel		      “Test” Checked - Not illuminated

Throttle				           Check Idle  “Actual ___”

Throttle						  Set 1000RPM

Throttle Friction Lock					       Adjust/Set

*****

Pre Take Off

Passenger Seat Backs				       Most Upright

Seats & Seat Belts				       Check Secure

Cabin Doors				             Closed & Locked

Fuel Quantity	(Gauges)				   Checked

Flight Controls				     Free & Correct

Flight Instruments				     Set & Checked

Heading Bug				       Runway Heading Set

Nav/GNSS(GPS) Switch (NAV or GPS)	                        Set

GNSS(GPS)	 ( Verify ON – “ENTER” + Validate)            Set

Radios  			      	                     Tuned & Set

NavAids 					          Tuned & Set

Auto Pilot				          Checked then, OFF

Mixture						         Rich

Wing Flaps				      Set (Zero/10 degrees)

Trim							   Checked

Trim		                   (Take-Off position) Set for Take-Off

Fuel Selector						         Both

Clearance				   Received/Not Required

Course Bar/Track Bar (HSI) (if fitted)			Set

Brakes						        Release/OFF

*****

Hold Point

Hatches							      Closed

Clearance to cross			       Received/Not required

Lookout (RWY, FINAL, BASE L&R)			  Complete

*****

Line Up

Transponder						 Set/ALT

Pitot Heat						         ON

Strobes							         ON

Runway Alignment					 Checked

DI/DG (HSI)				     Aligned with Compass

Wing Flaps				       Set (Zero/10 degrees)

Windsock						 Checked

*****

Take Off

“Clear for Take-Off”			            Landing Light ON

Heels						        On the Floor

Throttle					       Full Open/ Max RPM

T’s & P’s 						      Green

Airspeed					            Increasing

*****

After Take Off

Brakes							     Apply

Wing Flaps						Retracted

Mixture							Full Rich

Throttle						            Full Open

T’s & P’s 						      Green

*****

Climb

Power						  (MAX)    Set

OAT						         Checked

Suction/ Vacuum				             Green

T’s & P’s  ( Engine)			           Checked/Green

Altimeter					         Checked

*****

Top of Climb

Level Off 				       Assigned Altitude

Auto-Pilot					 	ON

OAT						         Check

“Revised Freezing Level is..................”         (2deg. per 1000ft)

Set ASI					              OAT & ALT

Cowl Flaps 				  Closed or Not fitted

Set Power			  	      Ref. Cruise Table

Mixture Set					         (9 gph)

Check Fuel Pump					OFF

Top of Climb Time				        Logged

*****

Cruise

Compass			            Aligned to DG/DI/HSI

Correct Heading

Log				          Complete/Not Required

Engine				     T’s & P’s Green/Power Set

Mixture Leaned

Altimetry			   Checked QNH/Cruise Level

Radio				     Set/Call/Transponder Code

Orientation					         Checked

Fuel		       Sufficient/Appropriate tank/Log Complete

Forced Landing					    Select Field

*****

Pre - Manoeuvre

Height			   Sufficient to recover by 3000ft AGL

Airframe				  Sound & Configured

Security					        Checked

Engine						        Checked

Location					         Suitable

Lookout					       Complete

*****

Diversion

Details - log Time over known position or DR fix

Identify – confirm position, actual  location/fix & Destination

VNC/VTC/ERC- check along new track for P/R/D CTA/CTR

Estimate- Track & Heading, Distance & Time, Endurance

Report- R/T – “Centre-Flightwatch” FPlan amendment details

TAF- divert to aerodrome (TAF, NOTAM, Ops.Procedures)

*****

Top of Descent/ Pre Approach

Compass (DG)					            Aligned

Altimeter			     Assigned ALT Set/QNH Set

Aids (incl GNSS)			       Tuned & Identified

Approach Brief (IFR-Plate) &/or  Join Intentions		  Given

Mixture					          Full Rich

Radio(s)						      Set

*****

Pre Landing

Brakes					Checked/Heels on floor

Park Brake 	    OFF

Under carriage					     Fixed/Down

Mixture 					          Full Rich

Fuel Selector						    Both

Fuel Quantity Checked

Instruments (Engine T’s & P’s)	                         Checked/Green

Switches (Key- Magnetos)		          Confirm on Both

Carburettor heat				        Not Fitted

Hatches/Harnesses				             Secure

Autopilot						    OFF

Landing light						      ON

Traffic	 (Downwind/Base L&R/Final/Runway/Radio)  Checked

*****

Finals

Pitch							  Fixed

Undercarriage					     Fixed/Down

Flaps							      Set

Landing Clearance 			                       Obtained

Runway						   Clear

Windsock 					         Checked

*****

After Landing (vacating Runway)

Transponder					           Standby

Pitot heat						   OFF

Strobes					        OFF/As Required

Unnecessary lights (Landing Light)			   OFF

Flaps					 Identified & Retracted

Trim			            Set (Take-Off position) Neutral

SAR (Flight Watch)			  	        Cancelled

*****

Shut Down

Throttle							  1000RPM

Park Brake						      Set/ON

Magnetos				                Check Right/Left

Electrical Switches (Taxi Light)				           OFF

Avionics Master					           OFF

Mixture			(Pull FULL OUT)          ICO - Idle Cut OFF

Nav Lights 						           OFF

Beacon Light 						            ON

Ignition	Switch (Key)/Magnetos			        OFF/Keys Out

Fuel					      Gauges/Check Quantity

Master Switch						           OFF

Fuel Tanks					   Select Right Tank

*****

Vacating Aircraft

Flight Log						   Complete

Control Lock						        Set/IN

Throttle/Mixture					      Locked

Seat Belts						     Dressed

Fuel Tanks						            DIP

Doors (Cabin & Baggage)				      Locked

Air Inlet Bungs						         Fitted

Propeller 					 Dressed & Secured

Pitot Cover							ON

Tie downs 							ON

Post (After) Flight Inspection/External			    Checked

*****

Please Note

If you are the Last flight of the day the following applies:

Aircraft							Hosed OFF

Aircraft							    Chamois

Leading Edges				       Wiped Clean/NO Bugs

Windscreen						    Chamois

Throttle/Mixture Lock						ON

Aircraft Cover							ON

Gust Locks						         Fitted

*****

Information Page Only

Quick Reference Data

Vne							163KIAS

Vno							129KIAS

Va		     99KIAS (1111kg)  92KIAS (907kg)  82KIAS (725kg)

Vfe					        Under 110, Flaps 10

Under 85, Flaps 20,30

Vr							 55KIAS

Vs1							 44KIAS

Vs0							 33KIAS

Vx							 60KIAS

Vy							 79KIAS

Normal Climb (Take off)			         70-80KIAS

Short Field (Take off Flaps 10)				 57KIAS

Cruise Climb					         75-85KIAS

Landing Approach/Normal (Flaps up)		         65-75KIAS

(Flaps 30)		         60-70KIAS

Short Field (flaps 30)			 62KIAS

Best Glide (Clean)					 65KIAS

(Flaps Extended)				 60KIAS

Engine

Textron Lycoming				       IO-360-L2A

Rated Power					  160 Bhp rating

Maximum Engine RPM				          2400RPM

Oil (sump)				          Min 5 Max 8 USQ

Oil Pressure			       Max 115 psi/Idle Min 20 psi

Oil Temperature			        Max 245°F (118°C)

Propeller	           MacCauley 1C235/LFA7570,75”Diameter

Static RPM at full throttle		       Min 2065-2165RPM

Information Page Only

Fuel

Fuel Grade			 100LL Grade Aviation Fuel (Blue)

100 Grade Aviation Fuel (Green)

Total (2 Tanks)					 (56USG) 212 litres

Useable total (2 Tanks)				 (53USG) 200 litres

Planned Burn Rate (OM ref: B1-G)		(9 USGPH)   34 lph

Endurance:	           200 litres at 34 lph – (5hr : 50 mins) 350 mins

Weights

MRW						  1114kg (2457lb)

MTOW						  1111kg (2450lb)

MLW						  1111kg (2450lb)

BEW (see POH for each aircraft)		    744kg (1639lb)

Maximum Baggage Area 1			        54kg (120lb)

Area 2			          22kg (50lb)

Combined Baggage (Area 1 + Area 2)	        54kg (120lb)

Airframe

Length							       8.28m

Height (Top of Fin)					       2.71m

Height (Propeller tip)					             xx

Propeller Clearance					     28.5cm

Wing Span						       11.0m

Wing Area					                 16.17m2

Wing Loading					            68.71kg/m2

Tailplane Span						       3.45m

Min Turning Radius					       8.37m

Power Loading						6.94kg/hp

G limits	 (load factor)			        +3.8g/-1.52g (normal)

@ MTOW 2450lbs		                 +3.0g/-0.0g (flaps down)

Information Page Only

Systems

Stall Warning Horn				5~10kts before stall

Landing Gear					      Nose 5 in, 34psi

Main 6 in, 28psi

Nose Wheel Steering				     xx° Left & Right

Alternator						    28V/60A

Battery						       24V/xx Amp hr

Vacuum					            4.5’~5.5’ Hg

Starter Duty Cycle			         30 sec ON/ 2 min OFF

Circuit

Take Off					       FULL POWER

Upwind					         Full Power 70-80KIAS

Crosswind				         Full Power 70-80KIAS

Downwind			           2000-2200RPM/ 90-110KIAS

Base						 1500RPM/ 75KIAS

Landing Approach/Normal (flaps up) 	         1500RPM/ 65-75KIAS

(flaps 30) 	         1500RPM/ 60-70KIAS

Maximum Crosswind component			      15Knots

SLOW & LOW FLIGHT:      2200RPM – flap 20 deg – 80 KIAS

(CAUTION: Max 85 KIAS)

Information Page Only

PAX Brief

CAO 20.11, paragraph14 		OM Reference: A4.10

- Smoking 				h.    Dangerous Goods

- Seatbelts				i.     Operation of Doors

- Emergency Exits			j.    PAX in control seat

- Oxygen				k.    Aircraft Fire on ground

- Flotation Devices			l.     Fire Extinguisher

- Stowage of Hand Luggage		m.   First Aid Kits

- Special Survival equipment		n.    PAX Safety Briefing card

o.    Climate Control

p.    Headset use

q.    Comfort (Sick) bags

r.    Venues – Destinations

Example Brief:

“Welcome Aboard,

Just like airlines I am obliged to give you an emergency briefing so please pay attention.  This is a C172 Aircraft.  In the event of an emergency follow my instructions. There is a PAX Safety Briefing Card in the back pocket in each seat, please take time to review it.

The emergency exits are the doors that we enter the aircraft through.  They lock and unlock like this (demonstrate).  There is a baggage door however that is locked during flight.

The windows can be opened on the ground and they can be open as we taxi for airflow comfort, however we would close them and secure latches before take off.

There is no smoking at any stage on this flight.

Please wear seatbelts during takeoff and landing, or as I direct.  I would advise you to keep them on at all times.  They operate like this (demonstrate).

Any hand luggage should be in the baggage compartment or secure on the floor.

This aircraft has dual controls (demonstrate control column & rudder).  Please be sure NOT to touch the controls at any time.

Fire Extinguisher is located between the front seats and a first-aid kit is located in the baggage compartment.

Oxygen (if not fitted – do NOT mention)

Life jackets, floatation devices and Special Survival Equipment (if not carried do NOT mention)

A portion of the flight will be over water.  Life jackets are located behind the seats (or demonstrate how they are put on & worn, around the waist or over the head)

Any questions please don’t hesitate to ask.  Thank you & enjoy your flight.”

Information Page Only

Pre take-off safety Brief

Pilot to Pilot Only

“This will be a (normal/ short field) take off.

Flap setting will be (zero/ 10 degrees)

Rotate speed will be 55kts

Climb out speed will be ____kts

Engine failure on the Runway, I will...

- Close throttle & abort the take-off

- Brake

- Stopping on the remaining runway

- Shut down”

Engine failure airborne with SUFFICIENT runway remaining, I will...

- Lower the nose

- Close the throttle

- Land on the remaining runway or overshoot”

Engine failure with INSUFFICIENT runway remaining, I will...

- Lower the nose to the glide attitude

- Select a clear area left or right

- Close the throttle

- Use flaps as required to land in the selected area”

Information Page Only

After Start

Pilot		“Intercom Check”

Instructor	“Reading 1~5”

PAX		“Loud & Clear”

Taxi

Brakes

Pilot Taxiing

- “Brakes Checked, you have control to check brakes”  OR  “Brakes Checked, handing over for you to check brakes”

Instructor taxiing

- “Brakes checked, you have control”

Flight Instruments & NAVAIDS function check (left & right turn)

- “Turning Left, skidding right, AI Wings level, Compass/DI (DG, HSI) following, Navaid(s) tracking”

- “Turning Right, skidding left, AI Wings level, Compass/DI (DG, HSI) following, Navaid(s) tracking”

Pre Takeoff - Flight Controls – Free & Correct

“Control column Full forward, Full left & Full right.

Full aft (back) Full left & Full right.”

OR

“Control column full forward, elevator trailing edge down, full left, aileron trailing edge up – check left & right, full right, aileron trailing edge up – check right & left.

Control column full aft (back), elevator trailing edge up, full left, aileron trailing edge up – check left & right, full right, aileron trailing edge up – check right & left”

Information Page Only

Pre take off

Flight Instruments                     Checked Left to Right, Top to Bottom

“Reading Zero, wings level, altimeter set & checked

Flag away/wings level, DI/DG (HSI) aligned with compass, Heading bug set, VSI reading Zero”

Takeoff

- Take off Power Applied

- Heels on the floor

“Max RPM, T’s&P’s Green, Airspeed increasing”

Work Cycles

Attitude	Lookout	Performance

C&D:		Aim Point	Attitude	Speed

S&L:		Aim Point	Attitude	Height

Finals:		Aim Point    Aspect    Centreline    Airspeed

Nav. Turning Points:	Wind    Heading    Altitude    Time

Information Page Only

ALA strip length		      1 knot = 0.5 metres per second

Example:          80KIAS for 20 seconds : Strip length = 800 metres

100KIAS for 15 seconds : Strip length = 750 metres

Departure Gradient ALA’s ( Estimation method)

GRADIENT 5% = 1:20 (1 in 20)

Place “1 metre high stick” at end of strip and pace back 20 metres sight from ground level over the top of stick to ensure obstacle clearance

Time to PNR (min) =  Safe End(min) X GS home

GS home  +  GS out

Dist to PNR =  Time to PNR  X GS out

60

Dist to CP (ETP) = 	Total dist X GS home

GS home + GS on from CP

PH = AD ELEV + 30x(1013 – QNH)

ISA = 15 – (2 X PH/1000)

DH = PH + ((OAT – ISA) X 120)

Information Page Only

International Morse Code

A • ―		B ― • • •	C ― • ― •	D ― • •

E •		F • • ― •	G ― ― • 	H • • • •

I • • 		J • ― ― ― 	K ― • ― 	L • ― • •

M ― ―		N ― •		O ― ― ― 	P • ― ― •

Q ― ― • ―	R • ― • 	S • • • 	T ―

U • • ― 	V • • • ― 	W • ― ― 	X ― • • ―

Y ― • ― ―	Z ― ― • •