# 1961 Cessna 172 Checklist – N8458X PREFLIGHT INSPECTION CABIN 1. Pilot's Operating Handbook, AROW -- AVAILABLE IN THE AIRPLANE 2. Control Wheel Lock -- REMOVE 3. Ignition Switch – OFF 4. Flaps -- 30°, CHECK for proper extension 5. Master Switch -- ON 6. Fuel Quantity Indicators -- CHECK QUANTITY 7. Stall Warning Opening – CHECK 8. Remove PITOT TUBE COVER 8. Lights – Beacon, Strobe, Nav, Taxi & Landing 9. Master Switch -- OFF 10. Fuel Selector Valve – BOTH EMPENNAGE 1. Rudder Gust Lock -- REMOVE 2. Control Surfaces -- CHECK freedom of movement and security 3. Antennas -- CHECK for security RIGHT WING Trailing Edge 1. Flap -- CHECK freedom of movement and security 2. Aileron -- CHECK freedom of movement and security & counter weights installed 3. Position Light – GREEN RIGHT WING Leading Edge 1. Fuel Tank Vent Opening -- CHECK for blockage RIGHT WING 1. Main Wheel Tire -- CHECK for proper inflation and security 2. Brakes -- CHECK for cracks, security and leaky fluid 4. Fuel Sump -- DRAIN small amount, CHECK FUEL, CHECK CLOSED 4. Cabin Air Intake -- CHECK for blockage NOSE 1. Engine Oil Level -- CHECK, do not operate with less than 6 quarts. Fill to 8 quarts for extended flight 2. Fuel Strainer -- DRAIN for 4 seconds, CHECK CLOSED 3. Propeller and spinner -- CHECK for nicks and security 4. Carburetor Air Filter -- CHECK for blockage 5. Nose wheel Strut and Tire -- CHECK for proper inflation and security 6. Static Source Opening -- CHECK for blockage 7. Clean windscreen LEFT WING Leading Edge 1. Pitot Tube Cover -- REMOVE and CHECK opening for blockage 2. Stall Warning Opening -- CHECK for blockage 3. Fuel Tank Vent Opening -- CHECK for blockage LEFT WING Trailing Edge 1. Flap -- CHECK freedom of movement and security 2. Aileron -- CHECK freedom of movement and security 3. Position Light – RED LEFT WING 1. Main Wheel Tire -- CHECK for proper inflation and security 2. Brakes -- CHECK for cracks, security and leaky fluid 3. Fuel Sump -- DRAIN small amount, CHECK FUEL, CHECK CLOSED 4. Cabin Air Intake -- CHECK for blockage CABIN 1. Master Switch -- ON 2. Flaps -- RETRACT 3. Radios -- ON and WORKING, listen for ATIS, get CLEARANCE (CRADS) 4. Radios -- SET 5. Radios -- OFF 6. Master Switch -- OFF 7. Elevator Trim -- TAKEOFF 8. Baggage Door -- CLOSED and LOCKED 9. Tie Downs – DISCONNECT BEFORE STARTING ENGINE 1. Preflight Inspection -- COMPLETE 2. Passenger Briefing -- COMPLETE 3. Seats, Belts, Shoulder Harnesses -- ADJUST and LOCK 4. Fuel Selector Valve -- BOTH 5. Radios & Electrical Equipment -- OFF 6. Circuit Breakers -- CHECK IN 7. Brakes -- TEST and SET STARTING ENGINE ABOVE FREEZING 1. Mixture -- RICH 2. Carburetor Heat -- COLD 3. Prime -- AS REQUIRED, IN and LOCKED 4. Throttle – PUMP ONCE THEN OPEN 1/2 INCH 5. Master Switch -- ON 6. Rotating Beacon -- ON 7. Propeller Area -- CLEAR 8. Ignition Switch -- START (RELEASE when engine starts) 9. Throttle -- ADJUST for 1000 RPM or less 10. Oil Pressure -- CHECK 11. Mixture -- LEAN for Max RPM's or RICH below 3000' 12. Radios, Avionics Power Switch -- ON 13. Transponder – ALT 14. Lights - ON (as required) TAXI 1. Check for obstacles 2. Attitude Indicator - stable 3. Turn Coordinator -- WORKING 4. Directional Gyro -- WORKING 5. Suction/Vacuum -- WORKING 6. Electrical (Ammeter) – CHECK BEFORE TAKEOFF 1. Parking Brake -- SET 2. Cabin Doors / Seatbelts -- CLOSED and LATCHED 3. Flaps & Trim Tab – Take-off setting 4. Flight Controls -- FREE and CORRECT 5. Flight Instruments -- SET including D.G., A.I., Altimeter, Radios & Nav/GPS 6. Fuel Selector Valve – BOTH 7. Throttle -- 1700 RPM a. Mixture -- LEAN for Max RPM's, RICH below 3000' b. Magnetos -- CHECK (RPM drop should not exceed 100 RPM on either magneto or 50 RPM differential between magnetos) c. Carburetor Heat -- CHECK (for RPM drop) d. Engine Instruments -- CHECK e. Ammeter -- CHECK f. Suction Gage -- CHECK 8. Flight Instruments -- SET including D.G., A.I., Altimeter, Radios & Nav/GPS 9. Throttle – Idle then 800 RPM 10. Throttle Friction Lock -- ADJUST 11. Parking Brake – RELEASE Cleared For Takeoff 1. Strobes – ON 2. Transponder – ALT 3. Time – COPY 4. Runway heading -- ON D.G. 5. Airspeed – ALIVE 6. Go / No-Go point determined CIGAR 1. Controls 2. Instruments 3. Gas 4. Attitude 5. Run-up TAKEOFF Normal Takeoff 1. Wing Flaps -- UP 2. Carburetor Heat -- COLD 3. Throttle -- FULL OPEN 4. Elevator Control -- LIFT NOSE WHEEL at 60 IAS 5. Climb Speed -- 80-90 KIAS (Vy – 77 IAS / Vx 65 IAS) Short Field Takeoff 1. Wing Flaps – 10 degrees 2. Carburetor Heat -- COLD 3. Brakes -- APPLY 4. Throttle -- FULL OPEN 5. Mixture -- RICH (Above 3000 feet, LEAN to obtain MAX RPM's) 6. Breaks -- RELEASE 7. Elevator Control -- SLIGHTLY TAIL LOW 8. Climb Speed -- 65 IAS (Until all obstacles are cleared) 9. Wing Flaps -- RETRACT slowly after reaching 70 IAS Soft Field Takeoff 1. Wing Flaps – 10 degrees 2. Carburetor Heat -- COLD 3. Brakes -- APPLY 4. Throttle -- FULL OPEN 5. Mixture -- RICH (Above 3000 feet, LEAN to obtain MAX RPM's) 6. Breaks -- RELEASE 7. Elevator Control -- SLIGHTLY TAIL LOW 8. Climb Speed -- 65 IAS (Until all obstacles are cleared) 9. Wing Flaps -- RETRACT slowly after reaching 70 IAS ENROUTE CLIMB 1. Vy – 77 IAS / Vx – 65 IAS Airspeed -- 80-90 IAS (normal) 2. Throttle -- FULL OPEN 3. Mixture -- RICH below 3000', LEAN for MAX RPM's CRUISE 1. Power – 2100 – 2650 RPM 2. Trim Tab – Adjust 3. Mixture – Lean if above 3000’ PA BEFORE APPROACH - CGUMPS 1. Carb Heat – APPLY 2. Gas Fuel Selector Valve – BOTH 3. Undercarriage / Gear – DOWN and VERIFIED 4. Mixture – RICH (if applicable) 5. Propeller – FULL FORWARD 6. Safety - Seats, Belts, Harnesses - ADJUST and LOCK ===Lights ON=== APPROACH (Prior to IAF) 1. Missed Approach Review -- COMPLETE 2. ATIS -- ALTIMETER SETTING 3. Radios -- (NAV/COMM - IDENTIFY) 4. Time to MAP -- COPY 5. Heading Indicator -- SET 6. Altitude -- (MDA-30 sec, DH-50ft) At FAF or for Holding (6 T’s) 1. Turn -- AS NEEDED 2. Time -- ONE MINUTE 3. Twist -- SET inbound course 4. Throttle -- SET to holding speed 5. Talk -- AS NEEDED 6. Track – check heading & track PLEASE RETURN TO: Larry D Bass 303-944-2277 Pilot2277@gmail.com LANDINGS CGUMPS Again 1. Carb Heat – APPLY 2. Gas Fuel Selector Valve – BOTH 3. Undercarriage / Gear – DOWN and VERIFIED 4. Mixture – RICH (if applicable) 5. Propeller – FULL FORWARD 6. Safety - Seats, Belts, Harnesses - ADJUST and LOCK - Lights ON Normal Landing 1. Fuel Selector – BOTH ON 2. Mixture – Adjust if necessary 3. Airspeed -- 70-80 IAS (Flaps UP) 4. Carb Heat - Apply 5. Throttle - Reduce 6. Wing Flaps -- AS DESIRED (below 100 IAS) 7. Airspeed -- 65-75 IAS (Flaps DOWN) 8. Touchdown -- MAIN WHEELS FIRST 9. Landing Roll -- LOWER NOSE WHEEL GENTLY 10. Braking -- MINIMUM REQUIRED Balked Landing 1. Throttle -- FULL OPEN 2. Carburetor Heat -- COLD 3. Wing Flaps -- Retract to 10° 4. Airspeed -- 60 IAS 5. Wing Flaps -- RETRACT (slowly at 70 IAS) Short Field Landing 1. Base Leg – 60 – 65 IAS / Flap 30 degrees 2. Final – Full Flaps 3. Glide Path – clear obstacle 4. After Touchdown a. Flaps up b. Max braking c. Control wheel full aft Soft Field Landing 1. Normal approach configuration 2. Flare – Nose high attitude 3. Power – Add during flare 4. Rollout – power idle, increase back elevator 5. Braking – none during roll-out AFTER LANDING 1. Flaps – UP 2. Carburetor Heat – COLD 3. Mixture - Lean 4. Accessories -- OFF 5. Trim -- To Takeoff SECURING AIRPLANE 1. Radios -- OFF 2. Electrical Equipment, Lights – OFF 3. Nav & Lights – OFF 4. Mags – Check for ground 5. Mixture -- IDLE CUT-OFF 6. Mags – OFF 7. Strobe & Lights – OFF 8. Master Switch – OFF 9. Write - Tach / Hobbs time in record book 10. Flight plan – CLOSE TIE-DOWN AIRPLANE 1. Control Lock Installed 2. Pitot Tube Cover Installed 3. Tie-Downs Complete 4. Lock Cabin and Cargo Door EMERGENCY CHECKLIST ENGINE FAILURES -- During Takeoff Run 1. Throttle -- IDLE 2. Brakes -- APPLY 3. Wing Flaps -- RETRACT 4. Mixture -- IDLE CUT-OFF 5. Ignition Switch -- OFF 6. Master Switch – OFF Immediately after Takeoff 1. Airspeed -- 75 IAS (Flaps UP) or 70 KIAS (Flaps DOWN) 2. Mixture -- IDLE CUT-OFF 3. Fuel Selector Valve -- OFF 4. Ignition Switch -- OFF 5. Wing Flaps -- AS REQUIRED 6. Master Switch -- OFF 7. Land -- STRAIGHT AHEAD if below 400 feet AGL During Flight 1. Airspeed -- 75 IAS 2. Find place to land 3. Carburetor Heat -- ON 4. Throttle -- FULL 5. Mixture -- RICH 6. Fuel Selector Valve -- BOTH 7. Primer -- IN and LOCKED 8. Ignition Switch -- BOTH (or START if propeller is stopped) 9. Magnetos -- CHECK 10. Fuel Quantity Indicators -- CHECK QUANTITY 11. Radios -- Declare Emergency 121.5 mhz, Transponder -- SET to 7700 13. Landing -- EXECUTE FORCED LANDING FORCED LANDINGS -- Without Engine Power 1. Airspeed -- 75 IAS (flaps UP) or 70 IAS (Flaps DOWN) 2. Mixture -- IDLE CUT-OFF 3. Fuel Selector Valve -- OFF 4. Ignition Switch -- OFF 5. Wing Flaps -- AS REQUIRED (40° recommended) 6. Master Switch -- OFF 7. Doors -- UNLATCH PRIOR TO TOUCHDOWN 8. Touchdown -- SLIGHTLY TAIL LOW 9. Brakes -- APPLY HEAVILY Precautionary with Engine Power 1. Wing Flaps -- 20° 2. Airspeed -- 00 IAS 3. Selected Field -- FLY OVER, noting obstructions and terrain, then retract flaps upon reaching a safe altitude and airspeed. 4. Radios, Electrical Switches, and Radio Power Switch -- OFF 5. Wing Flaps -- 40° 6. Airspeed -- 60 KIAS 7. Master Switch -- OFF 8. Doors -- UNLATCH PRIOR TO TOUCHDOWN 9. Touchdown -- SLIGHTLY TAIL LOW 10. Ignition Switch -- OFF 11. Brakes -- APPLY HEAVILY Ditching 1. Radio -- TRANSMIT MAYDAY on 121.5 and SQUAWK 7700 2. Heavy Objects (in baggage area) -- SECURE or JETTISON 3. Approach -- High Winds, Heavy Seas -- INTO THE WIND Light Winds, Heavy Swells -- PARALLEL TO SWELLS 4. Wing Flaps -- 20° to 40° 5. Power -- ESTABLISH 300 FT/MIN DESCENT AT 60 IAS If no power : Flaps -- 10°, Airspeed -- 60 IAS or Flaps -- 0°, Airspeed -- 65 IAS 6. Cabin Doors -- UNLATCH 7. Life Vests -- Put ON, DO NOT inflate. 8. Touchdown -- LEVEL ATTITUDE AT 300 FT/MIN DESCENT 9. Face -- CUSHION at touchdown with folded coat 10. Airplane -- EVACUATE through cabin doors. If necessary, open windows and flood cabin to equalize pressure so doors can be opened. 11. Life Vests and Raft – INFLATE FIRES -- During Start On Ground 1. Cranking -- CONTINUE to get a start which would suck the flames and accumulated fuel through the carburetor and into the engine. If the engine starts: 2. Power -- 1700 RPM for a few minutes 3. Engine -- SHUTDOWN and inspect for damage If engine fails to start: 4. Throttle -- FULL OPEN 5. Mixture -- IDLE CUT-OFF 5. Cranking -- CONTINUE in an effort to get a start 6. Fire Extinguisher -- OBTAIN (have ground attendants obtain if not installed) 7. Engine -- SECURE a. Master Switch -- OFF b. Ignition Switch -- OFF c. Fuel Selector Valve -- OFF 8. Fire -- EXTINGUISH using fire extinguisher, wool blanket, or dirt 9. Fire Damage – INSPECT Engine Fire in Flight 1. Mixture -- IDLE CUT-OFF 2. Fuel Selector Valve -- OFF 3. Master Switch -- OFF 4. Cabin Heat and Air -- OFF (except wing root vents) 5. Airspeed -- 100 IAS (If fire is not extinguished, increase glide speed to find an airspeed which will provide an incombustible mixture) 6. Forced Landing – EXECUTE Electrical Fire in Flight 1. Master Switch -- OFF 2. All Other Switches (except ignition switch) -- OFF (includes radio power, pitot heat, radios, lights, and electrical equipment) 3. Vents/Cabin Air/Cabin Heat -- CLOSED 4. Fire Extinguisher -- ACTIVATE When fire is out: 5. Cabin -- VENTILATE If fire appears out and electrical power is necessary for continuance of flight: 6. Master Switch -- ON 7. Circuit Breakers -- CHECK for faulty circuit, do not reset 8. Radio Switches -- OFF 9. Radio Power Switch -- ON 10. Radio/Electrical Switches -- ON one at a time, with delay after each until short circuit is localized Cabin Fire 1. Master Switch -- OFF 2. Vents/Cabin Air/Cabin Heat -- CLOSED 3. Fire Extinguisher -- ACTIVATE When fire is out: 4. Cabin -- VENTILATE 5. Airplane -- LAND as soon as possible Wing Fire 1. Navigation Lights -- OFF 2. Strobe -- OFF 3. Pitot Heat Switch -- OFF 4. Airplane -- SIDE SLIP to keep flames away from cabin and LAND as soon as possible Landing With A Flat Main Tire 1. Wing Flaps -- AS DESIRED 2. Approach -- NORMAL 3. Touchdown -- GOOD TIRE FIRST, hold airplane off flat tire as long as possible with aileron control Static Source Blockage 1. Alternate Static Source Valve -- PULL ON 2. Airspeed -- Consult tables in POH ELECTRICAL POWER SUPPLY SYSTEM MALFUNCTIONS Ammeter Shows Excessive Rate of Charge 1. Generator -- OFF 2. Nonessential Electrical Equipment -- OFF 3. Flight -- TERMINATE as soon as practical Low-Voltage Light Illuminates During Flight 1. Radios, Avionics Power Switch -- OFF 2. Master Switch -- OFF 3. Master Switch -- ON 4. Low Voltage Light -- CHECK OFF 5. Radios, Avionics Power Switch -- ON If Low-Voltage Light illuminates again: 6. Generator -- OFF 7. Nonessential Electrical Equipment -- OFF 8. Flight -- TERMINATE as soon as practical..

004 Atlas Aviation, Inc.